CAUSAL5G

INVENTOR LAB NOTEBOOK

Causal Directed Acyclic Graph-Based Fault Isolation

for Cloud-Native 5G Standalone Core Networks

| Inventor | Krishna Kumar Gattupalli |
| --- | --- |
| Filing Status | US Provisional — Pending |
| Repository | github.com/gattupallik/causal5g |
| Notebook Version | 1.0 |
| Classification | CONFIDENTIAL — Attorney-Client Privileged |
| Last Updated | March 2026 |

PURPOSE OF THIS DOCUMENT

This notebook records the conception, development, and reduction to practice of the Causal5G invention.

It serves as a legal record for USPTO patent proceedings and inventorship determination.

| 1 | Invention Overview |
| --- | --- |

## 1.1 Invention Title

Causal Directed Acyclic Graph-Based Fault Isolation for Cloud-Native 5G Standalone Core Networks

## 1.2 Problem Statement

Cloud-native 5G Standalone (SA) core networks deploy Network Functions (NFs) as containerized microservices across multiple network slice instances simultaneously. When a KPI degrades in one or more slices, existing root cause analysis (RCA) tools cannot answer the operationally critical question:

Is the fault in a shared NF (affecting all slices that use it), or is it local to one slice's specific configuration, session state, or policy?

This distinction — NF-layer fault vs. slice-layer fault — is unique to 5G SA architecture and is not addressed by any prior art system. The consequences of misclassification are significant:

- Treating a slice-layer fault as NF-layer causes unnecessary NF restarts, disrupting all slices

- Treating an NF-layer fault as slice-layer delays remediation while the fault cascades across all slices sharing that NF

## 1.3 Core Invention Concept

Causal5G introduces a bi-level causal directed acyclic graph (DAG) that encodes the structural topology of a 5G SA core network at two levels simultaneously:

- Level 1: NF-layer graph — nodes are NF instances (AMF, SMF, UPF, PCF, NRF, AUSF, UDM), edges are derived from SBI HTTP/2 call sequences captured via the 5G core service mesh

- Level 2: Slice subgraphs — one subgraph per S-NSSAI, each a projection of Level 1 NF nodes, where a single NF node may appear in multiple subgraphs (shared NF) or only one (dedicated NF)

Causal discovery (PCMCI algorithm) runs over real-time telemetry — SBI metrics, PFCP N4 session statistics, and per-S-NSSAI KPIs — constrained by the 5G topology as a structural prior. Attribution scoring then determines whether the highest-scoring root cause node is shared (NF-layer) or isolated to one subgraph (slice-layer).

## 1.4 What Makes It Novel

The white space over all prior art (CauseInfer 2014, MicroCause 2020, MicroDiag 2021):

| Feature | Status in All Prior Art |
| --- | --- |
| Bi-level NF + slice subgraph DAG | Not present in any prior art |
| S-NSSAI slice identity in causal graph | Not present in any prior art |
| PFCP N4 session telemetry as causal input | Not present in any prior art |
| SBI HTTP/2 call graph as structural prior | Not present in any prior art |
| NF-layer vs. slice-layer fault disambiguation | Not present in any prior art |
| RAN→Transport→Core→Cloud hierarchy | Not present in any prior art |
| Closed-loop SBI/PFCP remediation | Not present in any prior art |

| 2 | Conception Log — Chronological Record |
| --- | --- |

This section records the chronological development of the invention from first conception through reduction to practice. Dates reflect conversation sessions with AI-assisted research. All inventive concepts originated with the inventor, Krishna Kumar Gattupalli.

## Phase 1 — Problem Identification & Prior Art Analysis

Session 1  Initial conception — 5G fault isolation problem identified

Inventor identified the core operational challenge: in multi-slice 5G SA deployments, existing monitoring tools (Prometheus, Grafana, Jaeger) can detect KPI degradation but cannot distinguish NF-layer from slice-layer root causes. This is the founding problem statement of the invention.

Session 2  Prior art landscape analysis

Systematic review of causal inference literature applied to network RCA:

  - CauseInfer (Chen et al., 2014) — PC algorithm for microservice RCA. Not 5G-aware, no slice topology.

  - MicroCause (Lin et al., 2020) — causal graph for microservice latency RCA. No SBI/PFCP awareness.

  - MicroDiag (Wu et al., 2021) — multi-metric RCA for microservices. Generic, no 5G NF modeling.

Conclusion: The causal graph + PC algorithm approach is prior art for generic microservices. The white space is 5G-specific: no prior art covers S-NSSAI slice topology, SBI interface call graphs, or PFCP session bindings as causal graph inputs.

## Phase 2 — Architecture Design

Session 3  Bi-level DAG architecture conceived

Inventor conceived the bi-level graph structure as the core architectural solution:

  - Level 1 NF graph with SBI-derived edges solves the general NF dependency problem

  - Level 2 slice subgraphs as projections solve the NF-shared vs. slice-dedicated disambiguation

  - PFCP N4 session statistics as a third telemetry source adds user-plane session-level causality

This session established the foundational claim structure: the bi-level DAG with SBI prior and PFCP inputs is the core novel element.

Session 4  Carrier deployment gap analysis — AT&T, T-Mobile, Verizon

Analysis of real carrier deployments confirmed the practical relevance:

  - AT&T FirstNet slices share AMF/PCF infrastructure — exact shared NF scenario the invention targets

  - T-Mobile 5G SA uses dedicated UPF per slice — dedicated NF scenario

  - Verizon mmWave SA deployments use N4 PFCP session steering — PFCP telemetry scenario

This analysis validated that the bi-level DAG design maps directly to real production architectures.

## Phase 3 — Claim Drafting (Version 1)

Session 5  Independent and dependent claims — v1 drafted

First full claim set drafted:

  - Independent Claim 1: Slice-topology-aware causal DAG construction + NF/slice root cause isolation

  - Dependent Claim 2: Hierarchical domain graph (RAN → Transport → Core → Cloud)

  - Dependent Claim 3: Closed-loop remediation action mapper

Algorithm decision: algorithm-agnostic language in Claim 1 (broad independent claim) with PCMCI named in dependent Claim 4 (enablement). Rationale: PCMCI is best for time-series telemetry (autocorrelation-aware) but naming it only in a dependent claim preserves broad independent claim scope.

## Phase 4 — Claim Refinement & Title Update (Version 2)

Session 6  Title terminology updated — virtualized → cloud-native

Inventor identified that 'virtualized' is legacy EPC-era terminology not aligned with current 5G industry framing. Research confirmed:

  - Ericsson, Nokia, T-Mobile, Samsung all use 'cloud-native' / 'CNF' in 5G core product naming (2024-2025)

  - 3GPP SA6, CNCF telecom working group, and USPTO 5G patents use 'cloud-native 5G Standalone' as standard framing

  - Ericsson WO2025216683A1 (April 2025) uses 'cNF' (containerized NF) terminology — confirms CNF/cloud-native as current standard

Final title selected: Causal Directed Acyclic Graph-Based Fault Isolation for Cloud-Native 5G Standalone Core Networks.

Session 6  System/apparatus claim added (Claim 4)

Added independent apparatus/system claim to complement method Claim 1. Claim 4 additionally names PCMCI specifically with time-lag edge annotation, providing: (a) two independent claim types for prosecution strategy, (b) specific algorithm enablement for §112 compliance.

Session 6  Figure descriptions drafted

Four figure descriptions drafted for patent illustrator:

  - Fig. 1A — Level 1 NF-layer causal DAG with SBI edges and attribution heatmap

  - Fig. 1B — Level 2 slice subgraph projections per S-NSSAI with shared node markers

  - Fig. 2 — Four-domain hierarchical graph with cross-domain propagation example

  - Fig. 3 — Closed-loop remediation sequence flow with effectiveness verification

Session 6  Key term definitions (§112 compliance) drafted

Full set of §112 written description definitions drafted for: S-NSSAI, Network Slice, Slice Subgraph, SBI, SBI Topology Prior, PFCP, N4 Interface, PFCP Session Binding, Causal DAG, Conditional Independence-Based Causal Discovery Algorithm, Causal Attribution Score, NF-Layer Root Cause, Slice-Layer Root Cause, PCMCI.

| 3 | Patent Claim Set — Final Version |
| --- | --- |

The following claims are as filed in Patent Draft v2.0. All four claims are recorded here verbatim for inventorship and conception date purposes.

## Claim 1 — Independent Method Claim

### Slice-Topology-Aware Causal DAG Construction and NF/Slice Root Cause Isolation

1. A computer-implemented method for root cause analysis in a cloud-native 5G standalone (SA) core network, the method comprising:

maintaining, by a monitoring system, a bi-level causal directed acyclic graph (DAG) representing a 5G core network topology, wherein: a first level comprises nodes corresponding to individual 5G Network Functions (NFs) including at least an AMF, SMF, UPF, and PCF, with directed edges derived from SBI HTTP/2 call sequences captured via a 5G core service mesh; and a second level comprises slice subgraphs, each corresponding to a distinct network slice instance identified by an S-NSSAI value, wherein nodes are projections of first-level NF nodes, and a single NF node may be shared across multiple slice subgraphs;

receiving, in real time, telemetry streams comprising at least: NF-layer performance metrics via 3GPP Nnf service operations; PFCP session-level statistics from N4 interface sessions between SMF and UPF instances; and per-slice KPIs including PSESR, user-plane latency, and packet loss ratio per S-NSSAI;

constructing, using a conditional independence-based causal discovery algorithm over a sliding time window, a causal graph constrained by the bi-level topology as a structural prior, limiting candidate edges to SBI producer-consumer pairs or PFCP session bindings;

isolating a root cause by: identifying whether attribution scores concentrate on a shared NF node (NF-layer root cause) or within a single slice subgraph while peer slices remain unaffected (slice-layer root cause); and

generating a root cause report specifying: root cause type, attribution score, affected S-NSSAI identifiers, and implicated N4 PFCP session or SBI service operation.

## Claim 2 — Dependent Method Claim

### Hierarchical Four-Domain Graph for Scale

2. The method of Claim 1, further comprising: organizing the bi-level causal DAG within a four-domain hierarchical graph comprising: (a) RAN domain layer with gNB/CU/DU nodes and radio KPIs; (b) transport domain layer with N2/N3/N9 backhaul nodes and latency/jitter metrics; (c) core domain layer comprising the bi-level structure of Claim 1; and (d) cloud infrastructure layer with VM/container/Kubernetes pod nodes per NF instance; wherein causal discovery runs independently per domain and cross-domain edges are inferred via conditional independence tests on boundary metrics.

## Claim 3 — Dependent Method Claim

### Closed-Loop Remediation Action Mapper

3. The method of Claim 1, further comprising: maintaining a remediation action policy store associating root cause signatures with actions (NF scale-out, PFCP re-establishment, PCF admission tightening, SMF traffic steering); upon root cause isolation, automatically selecting and executing the applicable action via control-plane API calls; and verifying effectiveness by re-evaluating attribution scores post-remediation and alerting if scores remain above threshold.

## Claim 4 — Independent System/Apparatus Claim

### System with PCMCI Named + Time-Lag Edge Annotation

4. A system for root cause analysis in a cloud-native 5G SA core network, comprising one or more processors and memory storing instructions to: maintain the bi-level causal DAG of Claim 1; receive real-time telemetry of Claim 1; execute the PCMCI causal discovery algorithm with MCI tests over tau_max time lags constrained by the topology structural prior, producing a time-lagged causal DAG with per-edge lag annotation; isolate root cause per Claim 1; and generate a root cause report additionally specifying the time-lag of the highest-scoring causal edge.

| 4 | Repository Commit Log — Reduction to Practice |
| --- | --- |

Repository: git@github.com:gattupallik/causal5g.git

All code changes committed via git. Commit hashes serve as timestamped evidence of reduction to practice for each claim element.

| Commit Hash | Date | Description & Claims Enabled |
| --- | --- | --- |
| f1aa98f | Mar 2026 | Initial repository creation — project structure established |
| 838f722 | Mar 2026 | patent: Patent Draft v2.0 — title updated to Cloud-Native 5G Standalone (Claims 1-4) |
| 3fe642e | Mar 2026 | feat: 14 enablement module stubs — full §112 enablement trail for Claims 1-4 |
| latest | Mar 2026 | feat: pyproject.toml + README (technical disclosure) + 40+ unit tests (Claims 1, 3, 4) |

## 4.1 Enablement Checklist — Claim Element to Code Module Mapping

Every novel claim element maps to a committed code module. This table is the §112 enablement trail.

| Claim Element → Module | Claim |
| --- | --- |
| bilevel_dag.py — Bi-level causal DAG (NF + slice subgraph) | Claim 1 |
| topology_prior.py — SBI + PFCP structural prior constraint | Claim 1 |
| sbi_collector.py — SBI HTTP/2 call sequence capture | Claim 1 |
| pfcp_collector.py — PFCP N4 session statistics | Claim 1 |
| slice_kpi.py — Per-S-NSSAI KPI measurement (PSESR, latency, loss) | Claim 1 |
| discovery.py — Algorithm-agnostic causal discovery engine | Claim 1 |
| attribution.py — NF-layer vs slice-layer attribution scoring | Claim 1 |
| report.py — Structured root cause report generation | Claim 1 |
| hierarchical_dag.py — Four-domain RAN/Transport/Core/Cloud graph | Claim 2 |
| cross_domain.py — Cross-domain causal edge inference | Claim 2 |
| policy_store.py — Remediation policy lookup by root cause type | Claim 3 |
| executor.py — K8s/SMF/PCF control-plane API execution | Claim 3 |
| verifier.py — Post-remediation effectiveness verification | Claim 3 |
| pcmci.py — PCMCI time-lagged causal discovery (tigramite) | Claim 4 |

| 5 | Technical Architecture Record |
| --- | --- |

## 5.1 System Data Flow

The following describes the end-to-end data flow of the Causal5G system as implemented:

- Telemetry Ingestion: Three parallel collectors ingest real-time data — sbi_collector.py captures SBI HTTP/2 call sequences from the service mesh; pfcp_collector.py captures PFCP session statistics from N4 interface; slice_kpi.py captures per-S-NSSAI KPIs from NF exporters.

- Topology Prior Construction: topology_prior.py loads the 3GPP standard SBI producer-consumer relationship set and augments it with observed service mesh call graph edges and active PFCP session bindings.

- Bi-Level DAG Maintenance: bilevel_dag.py maintains the Level 1 NF graph and Level 2 slice subgraph registry. The DAG is updated as NF instances are added/removed and slice subgraph memberships change.

- Causal Discovery: discovery.py runs the configured backend (pcmci.py for PCMCI, or pluggable PC/FCI) over a sliding telemetry window, applying the topology prior as a structural constraint to limit the candidate edge search space.

- Attribution Scoring: attribution.py applies PageRank-based scoring on the reverse causal graph from the anomaly node, then classifies the top-scoring node as NF-layer (shared across slices) or slice-layer (isolated to one subgraph).

- Report Generation: report.py produces a structured RootCauseReport with all Claim 1 required fields.

- Closed-Loop Remediation (Claim 3): policy_store.py looks up the applicable remediation action; executor.py issues the control-plane API call; verifier.py re-evaluates attribution scores post-remediation.

## 5.2 Key Design Decisions & Rationale

### Algorithm: Algorithm-agnostic (Claim 1) + PCMCI named (Claim 4)

Rationale: Keeping Claim 1 algorithm-agnostic preserves maximum claim scope and avoids prior art on specific algorithms (PC algorithm is well-known). Naming PCMCI in Claim 4 (system claim) provides §112 enablement with the strongest time-series algorithm for network telemetry. PCMCI's MCI test handles autocorrelation in network KPI time series better than the standard PC algorithm's partial correlation test.

### Topology Prior: Structural constraint vs. pure data-driven

Rationale: Pure data-driven causal discovery over 5G telemetry would require enormous data volumes and would discover spurious edges (e.g., AMF-UPF direct edges that have no physical SBI relationship). The SBI topology prior encodes 3GPP domain knowledge as a whitelist of allowed edges, reducing the search space from O(N²) to O(E_SBI), dramatically improving discovery accuracy and speed for production deployments.

### Bi-level vs. single-level graph

Rationale: A single-level graph flattening all NF instances and all slice instances into one graph loses the structural information needed to distinguish NF-layer from slice-layer faults. The bi-level structure explicitly preserves the shared/dedicated NF topology, making the NF-layer vs. slice-layer classification a structural property of the graph rather than an inference problem.

### PFCP as causal input vs. SBI-only

Rationale: SBI metrics capture control-plane NF-to-NF call performance. PFCP N4 session metrics capture user-plane session state — packet loss, forwarding failures, session drops — which often manifest before control-plane SBI metrics degrade. Including PFCP as a causal input gives the system earlier fault signal for user-plane root causes, which is the majority of end-user-affecting faults in production 5G SA deployments.

| 6 | Prior Art Analysis |
| --- | --- |

## 6.1 CauseInfer (Chen et al., 2014)

System: PC algorithm-based causal graph for root cause analysis in internet services.

What it teaches: Applying conditional independence-based causal discovery to operational metrics for microservice RCA. Establishing the basic pattern of using causal DAGs to find root causes in distributed systems.

What it does NOT teach: 5G SA architecture, S-NSSAI slice identity, SBI HTTP/2 interface structure, PFCP session state, bi-level NF + slice graph, NF-layer vs. slice-layer disambiguation, cloud-native NF topology priors.

Distinguishing feature of Causal5G: All novel claim elements of Claim 1 are absent from CauseInfer.

## 6.2 MicroCause (Lin et al., 2020)

System: Causal graph for microservice latency RCA using PC algorithm with metric correlation.

What it teaches: Extending causal RCA to handle large numbers of microservice metrics efficiently via metric pruning before causal discovery.

What it does NOT teach: Any 5G-specific structure. Treats all microservices as equivalent nodes with no domain topology knowledge. No SBI, PFCP, S-NSSAI, or slice subgraph concept.

Distinguishing feature of Causal5G: The topology structural prior (SBI + PFCP) is the direct innovation over MicroCause's generic approach.

## 6.3 MicroDiag (Wu et al., 2021)

System: Multi-metric causal RCA for cloud microservices with hierarchical anomaly detection.

What it teaches: Hierarchical organization of metrics (infrastructure → application) for multi-layer RCA. Partial overlap with Claim 2's four-domain hierarchy concept.

What it does NOT teach: 5G slice topology, SBI/PFCP interfaces, S-NSSAI, NF-layer vs. slice-layer distinction. The hierarchical structure in MicroDiag is generic (infra vs. app) not 5G-specific (RAN/transport/core/cloud).

Distinguishing feature of Causal5G: Claim 2's four-domain hierarchy is specifically tailored to 5G SA domains with domain-appropriate telemetry granularities, which MicroDiag does not anticipate.

## 6.4 Ericsson WO2025216683A1 (April 2025)

System: NWDAF-assisted anomaly detection for 5G core NFs using cNF anomaly reporting.

What it teaches: Using NWDAF as a centralized anomaly analytics function in 5G SA. NF-to-NWDAF anomaly reporting. Remediation action triggering via OAM.

What it does NOT teach: Causal DAG construction, bi-level slice subgraph topology, PFCP session telemetry as causal input, SBI call graph as topology prior, NF-layer vs. slice-layer causal attribution. NWDAF in this patent is a correlation engine, not a causal discovery system.

Distinguishing feature of Causal5G: The causal directed acyclic graph + bi-level slice topology is the core differentiator. Ericsson's system detects anomalies; Causal5G isolates root causes with structural causal attribution.

| 7 | Filing Roadmap & Next Steps |
| --- | --- |

## 7.1 Immediate Actions (Before Provisional Filing)

- Commission patent drawings — Figures 1A, 1B, 2, 3 must be formal black-and-white USPTO line drawings from a certified patent illustrator. Current figure descriptions in Patent Draft v2.0 serve as the illustrator brief.

- Attorney review — Have a registered patent attorney review all four claims, particularly the structural prior constraint language in Claim 1 and the remediation group-of-alternatives in Claim 3.

- Complete executor.py — The three NotImplementedError stubs in remediation/executor.py (_scale_out_nf, _reestablish_pfcp, _tighten_pcf_admission) should be implemented before filing to strengthen §112 enablement.

- File US Provisional — File to lock in priority date. Provisional does not require formal claims but should include Patent Draft v2.0 and all figures.

## 7.2 Post-Provisional (12-Month Window)

- Formal prior art search — Commission a professional patent search targeting H04L41/0631 (RCA in networks), H04W24/02 (5G network monitoring), and H04L41/16 (AI in network management) CPC codes.

- US Non-Provisional filing — Must be filed within 12 months of provisional date.

- PCT application — File for international coverage: EPO (Europe), JPO (Japan), KIPO (Korea), CNIPA (China). All are significant 5G patent jurisdictions.

- CPC classification target — Primary: H04L41/0631. Secondary: H04W24/02, H04L41/16, H04W28/24 (network slicing QoS).

## 7.3 Prosecution Strategy Notes

- Claim 1 is intentionally algorithm-agnostic to survive obviousness attacks on specific algorithms (PC, PCMCI are known). The novelty is in the 5G-specific topology prior and bi-level structure, not the algorithm itself.

- If the examiner rejects on prior art grounds citing CauseInfer/MicroCause, the response should emphasize the SBI structural prior + PFCP telemetry + S-NSSAI slice subgraph as a combination not taught or suggested by any single reference.

- Claim 4 (apparatus) provides a fallback independent claim if Claim 1 is narrowed during prosecution. Having two independent claims in different claim types (method + apparatus) is standard prosecution strategy.

- The PCT filing in EPO should target the ETSI 5G standardization community as the primary commercial audience — Ericsson, Nokia, Samsung, Huawei all file extensively in this space.

## 7.4 Commercial Landscape

| Target Licensee | Relevance |
| --- | --- |
| Ericsson | Largest 5G core vendor globally; CNIS cloud-native platform; active patent filer in NF management |
| Nokia | 5G core cloud-native AirFrame platform; active in NWDAF-based analytics |
| Samsung Networks | 5G SA core deployments at T-Mobile, KDDI, Dish; cloud-native architecture |
| Mavenir / Amdocs | Open-source 5G core vendors; cloud-native NF deployments |
| AT&T / T-Mobile / Verizon | Direct operators with multi-slice SA deployments matching the invention scenario |
| Cisco | Cloud-native converged core gateway (4G/5G) with T-Mobile |

| 8 | Document & Artifact Index |
| --- | --- |

All artifacts related to this invention are recorded below for completeness of the inventor record.

| Artifact | Location / Description |
| --- | --- |
| Patent Draft v2.0 (DOCX) | patent/drafts/Causal5G_Patent_Draft_v2.docx — Full specification with 4 claims, figure descriptions, §112 definitions, prior art table, enablement checklist |
| Inventor Lab Notebook (this document) | patent/drafts/Causal5G_Lab_Notebook_v1.docx — Chronological conception record, claim set, architecture, prior art analysis, filing roadmap |
| Python Package | causal5g/ — 14 modules, fully installable via pip install -e '.[dev]' |
| Unit Test Suite | tests/ — 40+ pytest cases covering Claims 1, 3, 4 claim elements |
| Package Metadata | pyproject.toml — dependencies, extras, tool config |
| Technical README | README.md — Technical disclosure with architecture diagram and prior art table |
| commit f1aa98f | Initial repo creation |
| commit 838f722 | Patent Draft v2.0 committed |
| commit 3fe642e | 14 enablement module stubs committed |
| commit latest | pyproject.toml + README + unit tests committed |

This notebook should be printed, signed, and dated by the inventor and a witness. A signed physical copy is the strongest form of conception date evidence in USPTO proceedings. Digital copies with git commit timestamps provide corroborating evidence.

## Inventor Signature

| Krishna Kumar Gattupalli, Inventor | Date |
| --- | --- |

| Witness Name & Signature | Date |
| --- | --- |